Before following the steps, it helps to understand the little chain reaction inside the frame.

The instructions will walk you through the setup, but the mechanism is forgiving. Once you understand what each part is trying to do, you can use the steps as both a guide and a checklist.

How the mechanism works:

- Clock mechanism sits inside the frame.

- Capstan presses onto the clock mechanism.

- String ties to the capstan.

  - Then runs through the clock/string guide hole.

  - And through the top-left eyelet.

  - Then ties to the zipper anchor.

- Zipper head supports the latch.

- Latch supports the weight.

- Weight is tethered to the sliding cover image.

- Cover image sits over the reveal image.

When the timer runs, the clock winds the string, pulls the zipper, releases the latch, drops the weight, slides the cover image up, and reveals the image underneath.

Easy-to-miss checks:

[CODEX – program these as actual check-boxes user can check and uncheck.]
• Tuck the stem of the weight inside the roller lip so it cannot fall out.
• Fasten the trap-door latch with the C-clip.
• Thread the string through both the boss guide and the top-left eyelet.
• Pull any remaining slack above the string/clock guide.